360 Vision Technology

“Predator”

Fully functional PTZ camera

With IR Lamps Option

and

Thermal Imaging

Predator versions detailed within this document;

18x camera version

The fully functional rugged PTZ camera shall be a Predator PTZ camera rated to IP68 with a high resolution 18x true day/night camera.

Fitted with the new 360 Vision White Light and IR module, using the camera electronics to determine when to switch on, with user selectable levels via OSD

28x camera version

The fully functional rugged PTZ camera shall be a Predator PTZ camera rated to IP68 with a high resolution 28x true day/night camera.

Fitted with the new 360 Vision White Light and IR module, using the camera electronics to determine when to switch on, with user selectable levels via OSD

36x camera version

The fully functional rugged PTZ camera shall be a Predator PTZ camera rated to IP68 with a high resolution 36x true day/night camera.

Fitted with the new 360 Vision White Light and IR module, using the camera electronics to determine when to switch on, with user selectable levels via OSD

Thermal Imaging (when fitted)

9Hz thermal Imager version

The fully functional PTZ camera shall be a Predator PTZ Thermal camera, rated to IP68 with a 9hz Thermal Imaging camera, and be available with either a 336 line resolution

or a 640 line resolution FLIR Thermal imaging core.

The Thermal core shall be fitted with a 35mm lens, with an optional choice at time of build of either;		9mm

13mm

19mm

25mm

The Predator Thermal Camera will provide TWO independent outputs

Output 1 – Optical Camera

Output 2 – Thermal Camera

When a Predator is fitted with a thermal camera the ‘Thermal settings’ menu is used to set it up.

a Gain Mode.  The three settings are: ‘Automatic’ (default), ‘Low Gain’ and ‘High Gain’.

b Gain Control Mode.

The three settings are: ‘Histogram’ (default), ‘Linear’ and ‘Auto Bright’.

c Thermal Colour.  The twelve settings are: ‘White Hot’ (default), ‘Black Hot’, ‘Fusion’, ‘Rainbow’, ‘Glowbow’, ‘Ironbow 1’, 	‘Ironbow 2’, ‘Sepia’, Colour 1’, ‘Colour 2’, ‘Ice and fire’ and ‘Rain’.  The setting will be chosen by personal preference.

d Dual Addressing When a thermal camera is fitted, and the Predator is controlled by RS485) the Predator can use ‘dual addressing’ which allows telemetry commands to be received on two addresses. The controller can select either the optical or thermal camera for viewing and send telemetry to either.

Dual addressing can be set to ‘OFF’ (default) or ‘ON’.

When Dual Addressing is ‘ON’ the Predator will accept telemetry command for the ‘address’ that has been set using the Predator Set-up program and for ‘address + 1’.

Optical camera video = ‘address’ (This is the address set using Set-up program.

Thermal camera video = ‘address + 1’

The physical attributes of the unit shall meet the following design and performance

specifications:

- The unit shall be environmentally rated to IP68

- The unit shall operate from +60 degrees to -20 degree centigrade temperature variations      (-30° with optional heater), up to hurricane force wind conditions (above190mph winds) and underwater to a depth of 1 meter

- The unit shall be machined from 6mm thick solid aluminum

- The unit shall be classed as vandal resistant

The unit shall have a weather resistant coating in a choice of colors:

- The unit shall be black (RAL 9011)

- The unit shall be white (RAL 9003)

- The unit shall be cool grey (RAL 7035)

- The unit shall be mid grey (RAL 7001)

The unit shall be available in any colour dependant on customer requirement.

Note: Speak to our sales team for clarification and notification of any additional charge

The unit shall have an option of a TEN Year manufacturer’s Paint warranty;

Note: Speak to our sales team for clarification of available colour’s and notification of Terms and Conditions and additional charge

- The unit shall have the ability to be mounted upright or inverted

- The unit shall be no more that 7.5Kg in weight

- The unit shall comply with ROHS, CE standards

The unit shall have dimensions of

- 256(W) (Pan Sweep 392)mm

- 345(H) (Tilt Sweep 406)mm

- 259(D)mm

- The unit shall feature an optically perfect, flat viewing window

- All weatherproof seals within the unit shall last for the 3yr warranty period of the unit without need for maintenance or replacement

- The unit shall be supplied with attached 4 inch PCD base as standard with pre-drilled hole suitable for a safety chain

- The unit shall be capable of full operation at a composite cable lengths of up to 40 meters maximum between PTZ camera head and the power supply

- The unit’s pan and tilt system shall have a minimum performance or exceed the following

- design:

- The unit shall feature 3 Phase motors for its pan/tilt drive

- The pan drive speed shall range from 0.1º per second to 120º per second, 360° continuous rotation and absolute positioning

- The tilt drive speed shall range from 0.1º per second to 120º per second and absolute positioning

- The unit shall be capable of 180° Tilt coverage

- The unit shall be capable of 360º continuous rotation pan

- The unit shall utilise optical sensor positioning technology

- The unit shall have a absolute pan and tilt position accuracy

Connectivity of the unit shall meet the following design and performance specifications:

The unit shall have a purpose built composite cable available in the following lengths;

- 3m

- 10m

- 25m

- 40m

The composite cable contains video coax, power and data cables and shall be capable of full operation at a composite cable lengths of up to 40 meters maximum between PTZ camera head and the power supply

The unit shall have one end fitted with a IP68 12-way connector for direct connection to the Predator, the other end will be supplied with all cables stripped and tinned ready to be connected to the two–part connectors located within the Predator power supply.

IR70 Option

360 Vision IR70 Module shall meet the following design and performance specifications:

- Infrared illumination “IR70” shall be manufactured and provided by 360 Vision Technology Ltd

- The combined Infrared illumination “IR70 ” shall be provided to a range of 70 meters.

- The Infrared illumination “IR70” shall be provided at 850nm wavelength

- The Infrared illumination “IR70” shall be mounted to the head of the Predator giving directional illumination

- The Infrared illumination “IR70” shall be provided by twin combined IR illuminators

- The Infrared illumination “IR70” shall be rated to IP68

- The Infrared illumination “IR70” shall be constructed from case aluminum with Polycarbonate front window.

- The Infrared illumination “IR70” shall output an 30º beam angle

Patented “WHITE LIGHT” & IR with White Light Option

360 Vision IR40W Module shall meet the following design and performance

specifications:

- The combined White Light & Infrared illumination “IR40W” shall be manufactured and provided by 360 Vision Technology Ltd

- The combined White Light & Infrared illumination “IR40W” shall be provided to a minimum of 40 meters White Light & 70metres Infrared.

- The combined White Light & Infrared illumination “IR40W”” shall be provided at 850nm wavelength

- The combined White Light & Infrared illumination “IR40W” shall be mounted to the head of the Predator giving directional illumination

- The combined White Light & Infrared illumination “IR40W” shall be provided by combined WL & IR illuminators

- The combined White Light & Infrared illumination “IR40W” shall be rated to IP68

- The combined White Light & Infrared illumination “IR40W”” shall be constructed from case aluminum with Polycarbonate front window.

- The combined White Light & Infrared illumination “IR40W”” shall output an 30º beam angle

White Light Addendum

The camera can be fitted with a combined White Light/Infra-Red Light Unit to facilitate pre and post alarm on IR and White Light upon initiation and for the duration of the alarm, giving full colour pictures in hours of darkness.

The White Light Feature will have the following settings that can be adjusted remotely upon initiation of enabling of Preset 6:

New OSD Menu “Lamp Settings"

1. White Lamps – YES/NO - Default NO

2. White When Alarm – ON/OFF - Default ON

3. White Light Timeout – ON/OFF — Default ON

4. White Light Timeout (minutes) - 001 to 099 minutes - Default 1

Alarm Mode On from local alarm box will set ʻWhite When Darkʼ to ON

Alarm Mode Off from local alarm box will set ʻWhite When Dark' to OFF

- Preset 100 will force the white lights ON

- Preset 101 will force the white lights OFF

- if timeout enabled the white lights will only stay on for the timeout

timer value

“WHITE LIGHTING 0Nʼ is displayed as a warning whenever the white lights are on.

The camera module shall meet and/or exceed the following design and performance specifications:

18x camera module

- Shall feature a Sony 1/4" EXview™ “FCB-EX-490-EP” module

- Shall be a true day/night unit

- Shall switch from day mode (colour) to night mode (monochrome) via  the cameras module 360 vision software, it shall also be user selectable via preset levels, or via the Predators OSD Menus

- Shall have a resolution of 550TVL (PAL)

- Shall have an 18x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.7 lux – colour

- minimum illumination sensitivity of 0.05 lux – colour integration

- minimum illumination sensitivity of 0.01 lux – monochrome

- selectable on/off Backlight Compensation (BLC)

- 16 steps aperture control

- Automatic Gain Control (AGC) from -3 to -28 dB in 2dB steps

- Automatic mechanical IR cut-filter

- Signal to noise ration of greater than 50dB

- Internal / External (V-Lock) synchronization

- A lens that performs to 4.1mm to 73.8mm

- Angle of view of 48º (WIDE end) to 2.8º (TELE end) 8mm

- A shutter speed of 1/1 to 1/10,000s (22 steps)PAL

- 10 selectable settings that can be mapped to a preset on the remote keyboard, including shutter speed and colour control for ANPR applications

The camera module shall meet and/or exceed the following design and performance specifications:

28x camera module

- Shall feature a Sony 1/4" Exview HAD

- Shall be a true day/night unit

- Shall switch from day mode (colour) to night mode (monochrome) via  the cameras module 360 vision software, it shall also be user selectable via preset levels, or via the Predators OSD Menus

- Shall have a resolution of 550TVL (PAL)

- Shall have an 28x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.25 lux (50IRE) – colour

- minimum illumination sensitivity of 0.07 lux – colour integration

- minimum illumination sensitivity of 0.01 lux – monochrome

- Selectable on/off Backlight Compensation (BLC)

- 16 steps aperture control

- Automatic Gain Control (AGC) from -3 to -28 dB in 2dB steps

- Feature an automatic mechanical IR cut-filter

- A signal to noise ration of greater than 50dB

- Use Internal / External (V-Lock) synchronization

- A lens that performs to 3.5mm to 91.0mm

- An angle of view of 54.2º (WIDE end) to 2.2º (TELE end)

- A shutter speed of 1/1 to 1/10,000s (22 steps) PAL

- 10 selectable settings that can be mapped to a preset on the remote keyboard, including shutter speed and colour control for ANPR applications

The camera module shall meet and/or exceed the following design and performance specifications:

36x camera module

- Shall feature a 1/4" EXview™ HAD

- Shall be a true day/night unit

- Shall switch from day mode (colour) to night mode (monochrome) via  photocell integrated into the PTZ camera head that can be set to user selectable levels, or via the Predators OSD Menus

- Shall have a resolution of 550TVL (PAL)

- Shall have an 36x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 1.4 lux (50 IRE) – colour

- minimum illumination sensitivity of 0.1 lux – colour integration

- minimum illumination sensitivity of 0.01 lux – monochrome

- Selectable on/off Backlight Compensation (BLC)

- 16 steps aperture control

- Automatic Gain Control (AGC) from -3 to -28 dB in 2dB steps

- An automatic mechanical IR cut-filter

- A signal to noise ration of greater than 50dB

- Use Internal / External (V-Lock) synchronization

- A lens that performs to 3.4mm to 122.4mm

- An angle of view of 57.8º (WIDE end) to 2.7º (TELE end)

- A shutter speed of 1/1 to 1/10,000s (22 steps) PAL

- 10 selectable settings that can be mapped to a preset on the remote keyboard, including shutter speed and colour control for ANPR applications

The unit shall have the following operational design and performance specifications

- 360 telemetry presets

- Preset titles of 20 characters for each preset

- 4 programmable tours, each with 128 preset positions

- 4 learned patrols, each with up to 30 minute duration.

- 32 Privacy Zones

- Auto flip

- Proportional speed control

- Freeze presets

The camera will have a “Picture Freeze Facility" i.e. the on-screen visible image will have the facility to be frozen whilst the camera moves at high speed from preset to preset within a programmable guard tour allowing efficient DVR recording.

Home Function

- Preset 1 to 360

- Tours 1 to 4

- Mimics 1 to 4

- None

Engineering Servicing Menu

- Power Reset

- Factory Reset

The camera will be available in both an analogue and IP format, if the initially selected camera is analogue it must have the facility to be integrally upgraded to IP via a mere board change.

Colour Mono Changeover

- The camera will have three preconfigured colour/monochrome changeover modes and a user definable option.

The unit shall have the following electrical design and performance specifications:

Total power consumption of the standard camera shall be no more than 25.2W and no more than 50W when fitted with the heater option

Predator power supply providing +24V ac

An optional slim line power supply, for fitting in restricted spaces, providing +24V AC from the following input voltages 115V AC or 240V AC

Camera unit shall be fitted with control PCB’s including a control processor, voltage regulator, 3 Phase motor drives and connectivity of ancillary items such as heaters, infrared illuminators, washer/wiper and speakers

The unit shall feature the following software specifications:

Capable of either In Coax control or RS485 Twisted Pair Control

To operate with protocols other than 360 Vision’s proprietary software, programmed via a USB dongle.

- Pelco D 4800 protocol

- Pelco D 2400 protocol

- Pelco P 9600 protocol

- Pelco P 2400 protocol

- Pelco Coaxitron protocol

- VCL/Honeywell RS485 protocol

- VCL/Honeywell In Coax protocol

- BBV FSK protocol

- MME FSK protocol

- Ultrak protocol

- Forward Vision protocol

The unit shall have a range of options and accessories available for specific mounting and operational considerations:

- Suitable accessories shall be available for the unit to be mounted in an upright or inverted position, either on a base, from a ceiling, on a wall or on at the corner of a structure

Option to fit a washer nozzle to the 4” PCD base

- Option to fit a heater contained within the camera head & body

Optional integrated wiper with replaceable wiper blade

- Optional safety chain

Optional 8 input alarm card (incl. washer pump drive function) housed in the PSU

Optional washer pump drive card (no alarm inputs) housed in the PSU

The unit shall have the following support information and equipment shipped with it:

Comprehensive operation manual and installation manual

Comprehensive quick start guide