360 Vision Technology

“Predator”

fully functional ruggidised PTZ camera

with combined White Light & IR Lamps

Predator versions detailed within this document;

18x camera version

The fully functional rugged PTZ camera shall be a Predator PTZ camera rated to IP68 with a high resolution 18x true day/night camera.

Fitted with the new 360 Vision White Light and IR module, using the camera electronics to determine when to switch on, with user selectable levels via OSD

28x camera version

The fully functional rugged PTZ camera shall be a Predator PTZ camera rated to IP68 with a high resolution 28x true day/night camera.

Fitted with the new 360 Vision White Light and IR module, using the camera electronics to determine when to switch on, with user selectable levels via OSD

44x camera version

The fully functional rugged PTZ camera shall be a Predator PTZ camera rated to IP68 with a high resolution 44x true day/night camera.

Fitted with the new 360 Vision White Light and IR module, using the camera electronics to determine when to switch on, with user selectable levels via OSD

The physical attributes of the unit shall meet the following design and performance

specifications:

- The unit shall be environmentally rated to IP68

- The unit shall operate from +60 degrees to -40 degree centigrade temperature variations      (-50° with optional heater), up to hurricane force wind conditions (above190mph winds) and underwater to a depth of 1 meter

- The unit shall be machined from 6mm thick solid aluminum

- The unit shall be classed as vandal resistant

The unit shall have a weather resistant coating in a choice of colors:

- The unit shall be black (RAL 9011)

- The unit shall be white (RAL 9003)

- The unit shall be cool grey (RAL 7035)

- The unit shall be mid grey (RAL 7001)

The unit shall be available in any colour dependent on customer requirement.

Note: Speak to our sales team for clarification and notification of any additional charge

The unit shall have an option of a TEN Year manufacturer’s paint warranty;

Note: Speak to our sales team for clarification of available colour’s and notification of Terms and Conditions and additional charge

- The unit shall have the ability to be mounted upright or inverted

- The unit shall be no more that 8Kg in weight

- The unit shall comply with ROHS, CE standards

The unit shall have dimensions of

- 250(W) (Pan Sweep 420)mm

- 422(H) (Tilt Sweep 270)mm

- The unit shall feature an optically perfect, flat viewing window

- All weatherproof seals within the unit shall last for the 3yr warranty period of the unit without need for maintenance or replacement

- The MTBF of the Predator shall be 78,900hrs

- The unit shall be supplied with attached 4 inch PCD base as standard with pre-drilled hole suitable for a safety chain

- The unit shall be capable of full operation using a 360 Vision technology manufactured  12 core composite cable lengths of up to 40 meters maximum between PTZ camera head and the power supply

- The unit’s pan and tilt system shall have a minimum performance or exceed the following

- design:

- The unit shall feature 3 Phase motors for its pan/tilt drive

- The pan drive speed shall range from 0.1º per second to 120º per second, 360° continuous rotation and absolute positioning

- The tilt drive speed shall range from 0.1º per second to 120º per second and absolute positioning

- The unit shall be capable of 180° Tilt coverage

- The unit shall be capable of 360º continuous rotation pan

- The unit shall utilise optical sensor positioning technology

- The unit shall have a absolute pan and tilt position accuracy

Connectivity options of the Predator shall meet the following design and performance

specifications:

- Connections to Predator with 12way Pin Out

The Predator shall have a purpose manufactured composite cable available in the following lengths;

- 3m

- 10m

- 25m

- 40m

The composite cable contains video coax, power and data cables and shall be capable of full operation at a composite cable lengths of up to 40 meters maximum between PTZ camera head and the power supply

The unit shall have one end fitted with a IP68 12-way connector for direct connection to the Predator, the other end will be supplied with all cables stripped and tinned ready to be connected to the two–part connectors located within the Predator power supply.

Connections to Predators with HMA

Predator can be manufactured with a HMA (Hinged Mount Adaptor).

When manufactured with an HMA the ‘Predator Composite Cable’ is not used, the HMA allow the use of conventional cables.

| Video | RG59BU (low quality cable conductor) | 250m |
| --- | --- | --- |
|  | RG59BU (Copper central; conductor, RG11, CT125 | 1000m |
| Twisted Pair Data (RS485) | Beldon 9841, Beldon 9842, Beldon 8723 | 3000m |

Access to the connections are made by tilting the Predator.

A lanyard will be fitted at point of manufacture to the HMA.

24Vac Power connections between Predator PSU and Predator that is fitted with HMA

| Cable Size | Predator (No Lamps) | IR75 | IR100WL | IR125 | IR150SW | IR175 |
| --- | --- | --- | --- | --- | --- | --- |
| 0.75mm | 20m | 10m | 7.5m | 7.5m | 6m | NA |
| 1.00mm | 25m | 12m | 10m | 10m | 8m | NA |
| 1.25mm | 30m | 15m | 12.5m | 12.5m | 10m | NA |
| 1.50mm | 40m | 20m | 15m | 15m | 12m | 4m |
| 2.50mm | 65m | 30m | 25m | 25m | 20m | 6.5m |
| 3.00mm | 80m | 40m | 30m | 30m | 24m | 8m |
| 4.00mm | 100m | 50m | 40m | 40m | 32m | 10.5m |

Intelligent Lighting

IR75 Option

360 Vision IR 75 Module shall meet the following design and performance specifications:

- Infrared illumination “IR 75” shall be manufactured and provided by 360 Vision Technology Ltd

- The Infrared illumination “IR 75” shall be provided to a range of 75 meters.

- The Infrared illumination “IR 75” shall be provided at 850nm wavelength

- The Infrared illumination “IR75” shall be mounted to the head of the Predator giving directional illumination

- The Infrared illumination “IR75” shall be provided by twin combined IR illuminators

- The Infrared illumination “IR75” shall be rated to IP68

- The Infrared illumination “IR75” shall be constructed from case aluminum with Polycarbonate front window.

- The Infrared illumination “IR75” shall output an 30º beam angle

IR125 Option

360 Vision IR 125 Module shall meet the following design and performance specifications:

- Infrared illumination “IR125” shall be manufactured and provided by 360 Vision Technology Ltd

- The Infrared illumination “IR125” shall be provided to a range of 125 meters.

- The Infrared illumination “IR125” shall be mounted to the head of the Predator giving directional illumination

- The Infrared illumination “IR125” shall be provided by twin combined IR illuminators

- The Infrared illumination “IR125” shall be rated to IP68

- The Infrared illumination “IR125” shall be constructed from case aluminum with Polycarbonate front window.

- The Infrared illumination “IR125” shall output an 30º beam angle

IR175 Option

360 Vision IR 175 Module shall meet the following design and performance specifications:

- Infrared illumination “IR175 shall be manufactured and provided by 360 Vision Technology Ltd

- The Infrared illumination “IR175” shall be provided to a range of 175 meters.

- The Infrared illumination “IR175” shall be provided at 850nm wavelength

- The Infrared illumination “IR175” shall be mounted to the head of the Predator giving directional illumination

- The Infrared illumination “IR175” shall be provided by twin combined IR illuminators

- The Infrared illumination “IR175” shall be rated to IP68

- The Infrared illumination “IR175” shall be constructed from case aluminum with Polycarbonate front window.

- The Infrared illumination “IR175” shall output an 30º beam angle

Patented “WHITE LIGHT” IR100W Option

360 Vision IR 100W Module shall meet the following design and performance

specifications:

- The combined White Light & Infrared illumination “IR 100W” shall be manufactured and provided by 360 Vision Technology Ltd

- The combined White Light & Infrared illumination “IR 100W” shall be provided to a minimum of 100 meters White Light & 100metres Infrared.

- The combined White Light & Infrared illumination “IR 100W” shall be provided at 850nm wavelength

- The combined White Light & Infrared illumination “IR100W” shall be mounted to the head of the Predator giving directional illumination

- The combined White Light & Infrared illumination “IR 100W” shall be provided by twin combined WL & IR illuminators

- The combined White Light & Infrared illumination “IR 100W” shall be rated to IP68

- The combined White Light & Infrared illumination “IR 100W” shall be constructed from case aluminum with Polycarbonate front window.

- The combined White Light & Infrared illumination “IR 100W” shall output an 30º beam angle

Patented “WHITE LIGHT” IR150SW Option

360 Vision IR 150SW Module shall meet the following design and performance

specifications:

- The combined White Light & Infrared illumination “IR150SW” shall be manufactured and provided by 360 Vision Technology Ltd

- The combined White Light & Infrared illumination “IR150SW” shall be provided to a minimum of 125 meters White Light & 150metres Infrared.

- The combined White Light & Infrared illumination “IR150SW” shall be provided at 850nm wavelength

- The combined White Light & Infrared illumination “IR150SW” shall be mounted to the head of the Predator giving directional illumination

- The combined White Light & Infrared illumination “IR150SW” shall be provided by twin combined WL & IR illuminators

- The combined White Light & Infrared illumination “IR150SW” shall be rated to IP68

- The combined White Light & Infrared illumination “IR150SW” shall be constructed from case aluminum with Polycarbonate front window.

- The combined White Light & Infrared illumination “IR150SW” shall output an 30º beam angle

Black Light IR75 Option

360 Vision Black Light IR 75 Module (Covert Infra-Red) shall meet the following design and

performance specifications:

- Covert Infrared illumination “IR 75” shall be manufactured and provided by 360 Vision Technology Ltd

- The covert Infrared illumination “IR 75” shall be provided to a range of 75 meters.

- The covert Infrared illumination “IR 75” shall be provided at 940nm wavelength

- The covert Infrared illumination “IR75” shall be mounted to the head of the Predator giving directional illumination

- The covert Infrared illumination “IR75” shall be provided by twin combined IR illuminators

- The covert Infrared illumination “IR75” shall be rated to IP68

- The covert Infrared illumination “IR75” shall be constructed from case aluminum with Polycarbonate front window.

- The covert Infrared illumination “IR75” shall output an 30º beam angle

The camera module shall meet and/or exceed the following design and performance specifications:

18x camera module

- Shall feature a Sony 1/4"

- Shall be a true day/night unit

- Shall switch from day mode (colour) to night mode (monochrome) via  the cameras module 360 vision software, it shall also be user selectable via preset levels, or via the Predators OSD Menus

- Shall have a resolution of 670TVL (PAL)

- Shall have an 18x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.4 lux – colour

- minimum illumination sensitivity of 0.01 lux – monochrome

- selectable on/off Backlight Compensation (BLC)

- 16 steps aperture control

- ExView Technology

- Automatic Gain Control (AGC) from -3 to -28 dB in 2dB steps

- Automatic mechanical IR cut-filter

- Signal to noise ration of greater than 50dB

- Internal / External (V-Lock) synchronization

- A lens that performs to 4.1mm to 73.8mm

- Angle of view of 48º (WIDE end) to 2.8º (TELE end)

- 976(H) x 582(W) Pixels

- A shutter speed of 1/1 to 1/10,000s (22 steps)PAL

- 10 selectable settings that can be mapped to a preset on the remote keyboard, including shutter speed and colour control for ANPR applications

The camera module shall meet and/or exceed the following design and performance specifications:

28x camera module

- Shall feature a Sony 1/4"

- Shall be a true day/night unit

- Shall switch from day mode (colour) to night mode (monochrome) via  the cameras module 360 vision software, it shall also be user selectable via preset levels, or via the Predators OSD Menus

- Shall have a resolution of 670TVL (PAL)

- Shall have an 28x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.4 lux – colour

- minimum illumination sensitivity of 0.01 lux – monochrome

- Selectable on/off Backlight Compensation (BLC)

- 16 steps aperture control

- ExView Technology

- Automatic Gain Control (AGC) from -3 to -28 dB in 2dB steps

- Feature an automatic mechanical IR cut-filter

- A signal to noise ration of greater than 50dB

- Use Internal / External (V-Lock) synchronization

- A lens that performs to 3.5mm to 98.0mm

- An angle of view of 55.8º (WIDE end) to 2.1º (TELE end)

- 976(H) x 582(W) Pixels

- A shutter speed of 1/1 to 1/10,000s (22 steps) PAL

- 10 selectable settings that can be mapped to a preset on the remote keyboard, including shutter speed and colour control for ANPR applications

The camera module shall meet and/or exceed the following design and performance specifications:

44x camera module

- Shall feature a 1/4"

- Shall be a true day/night unit

- Shall switch from day mode (colour) to night mode (monochrome) via  photocell integrated into the PTZ camera head that can be set to user selectable levels, or via the Predators OSD Menus

- Shall have a resolution of 670TVL (PAL)

- Shall have an 44x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.6 lux – colour

- minimum illumination sensitivity of 0.01 lux – monochrome

- Selectable on/off Backlight Compensation (BLC)

- 16 steps aperture control

- ExView Technology

- Automatic Gain Control (AGC) from -3 to -28 dB in 2dB steps

- An automatic mechanical IR cut-filter

- A signal to noise ration of greater than 50dB

- Use Internal / External (V-Lock) synchronization

- A lens that performs to 3.06mm to 134.6mm

- An angle of view of 68.8º (WIDE end) to 1.6º (TELE end)

- 976(H) x 582(W) Pixels

- A shutter speed of 1/1 to 1/10,000s (22 steps) PAL

- 10 selectable settings that can be mapped to a preset on the remote keyboard, including shutter speed and colour control for ANPR applications

The Predator shall have the minimum following operational design and performance

specifications

- 360 telemetry presets

- Preset titles of 20 characters for each preset

- 16 Programmable tours *

- 4 learned patrols, each with up to 30 minute duration.

- 32 Privacy Zones

- Digital Zoom - Use this option to enable or disable digital zoom.

- White Balance - Change the camera white balance settings to suit the area.

- Line Lock - Used to line lock the camera.

- Line Phase - Can be used to manually adjust the line phase.

- Video Gain - Used to adjust the video level to suit different lengths of coax cable.

- Video Lift - Used to adjust the frequency of the video level to suit different lengths of coax cable.

- Wide Dynamic - Enable WDR

- Sunshield - Reduces tilt range of the camera for when a sunshield is fitted.

- Auto Flip - Use this option to set how the camera behaves when full tilt down is reached.

- Proportional - Automatically reduces/increases pan/tilt speed depending on zoom ratio.

- Freeze Presets - This option freezes the image, when moving between presets.

- Attack Detect - Camera will re-initialise camera motors so that camera looks back to the original position, if the camera is physically forced/moved. Default is on.

- Upright Mount - Can be used to flip the image, so the camera can be used on a pole or a ceiling.

*16 programmable tours, each with up to 90 preset positions

The tour of presets will be stored within the Predator.

Each tour point will consist of a preset number, dwell time and speed parameter.

Each tour can contain between 2 and 90 points.

Different points can use any preset number, including repeats of ones that have been used before.

The speed that the camera will travel at to reach the preset can be in the range 1°/sec (slowest) to 100°/sec (fastest).

The delay (dwell) at each point is defined in seconds, range 1 to 100.

Home Function

- Preset 1 to 360

- Tours 1 to 16

- Mimics 1 to 4

- None

Engineering Servicing Menu

- Power Reset

- Factory Reset

Colour Mono Changeover

- The camera will have three preconfigured colour/monochrome changeover modes and a user definable option.

Col/Mono Level - Col/Mono level settings can be changed to suit the site requirements.

Options are dark, medium and light. Default is dark. This sets when the camera goes into ‘mono mode’

- Mono On - Range is 7 to 25. A lower value will set the level on when the scene is brighter, a higher value will set the level on when the scene is darker.

- Mono Off - Range is 1 to 20. A lower value will set the level off when the scene is brighter, a higher value will set the level off when the scene is darker.

- Brightness - Can be used to artificially brighten the image, options are low, medium and bright.

- Bright Control - Options are automatic or preset seek activation

The camera will be available in both an analogue and IP format, if the initially selected camera is analogue it must have the facility to be integrally upgraded to IP via a mere board change.

The Predator shall have the facility to Map Presets

This is an option to re-map a function to a preset, should your controls

Options are:-

• none

• auto focus on

• auto iris on

• ir on

• wiper on

• white light

• fast shutter

• WDR

• ambient

• 20% lamp intensity

• 40% lamp intensity

• 60% lamp intensity

• 80% lamp intensity

• 100% lamp intensity

• video switch

• white day

• ir-mono

• mono

• normal night

• washer on

• washer off

The unit shall have the following electrical design and performance specifications:

Total power consumption of the standard camera shall be no more than 25.2W and no more than 50W when fitted with the heater option

Predator power supply providing +24V ac

An optional slim line power supply, for fitting in restricted spaces, providing +24V AC from the following input voltages 115V AC or 240V AC

Camera unit shall be fitted with control PCB’s including a control processor, voltage regulator, 3 Phase motor drives and connectivity of ancillary items such as heaters, infrared illuminators, washer/wiper and speakers

The unit shall feature the following software specifications:

Capable of either In Coax control or RS485 Twisted Pair Control

To operate with protocols other than 360 Vision’s proprietary software, programmed via a USB dongle.

- Pelco D 9600 protocol

- Pelco D 4800 protocol

- Pelco D 2400 protocol

- Pelco P 9600 protocol

- Pelco P 4800 protocol

- Pelco P 2400 protocol

- Pelco Coaxitron protocol (extended version)

- VCL/Honeywell RS485 protocol

- VCL/Honeywell In Coax protocol

- BBV/Dennard C FSK coax protocol

- Mark Mercer FSK coax protocol

- Ultrak protocol (RS485)

- Forward Vision protocol (RS485)

- Molynx RS485 protocol

- Molynx Coax protocol (Note: Only available when camera ordered with this requirement)

The unit shall have a range of options and accessories available for specific mounting and operational considerations:

- Suitable accessories shall be available for the unit to be mounted in an upright or inverted position, either on a base, from a ceiling, on a wall or on at the corner of a structure

Option to fit a washer nozzle to the 4” PCD base

- Option to fit a heater contained within the camera head & body

Optional integrated wiper with replaceable wiper blade

- Optional safety chain

Optional 8 input alarm card (incl. washer pump drive function) housed in the PSU

Optional washer pump drive card (no alarm inputs) housed in the PSU

The unit shall have the following support information and equipment shipped with it:

Comprehensive operation manual and installation manual (provided on CD)

Comprehensive quick start guide (provided on CD)

Intelligent Lighting - White Light/Infra-Red Light Addendum

The Predator can be fitted with a combined White Light/Infra-Red Light Unit to facilitate pre and post alarm on IR and White Light upon initiation of and alarm condition or by an Operator, providing colour pictures in hours of darkness.

White Light Feature will have the following settings that can be adjusted remotely upon initiation of enabling of Preset 6:

White Lamps - Set if white lights are to be used.

• White When Alarm - Set if white lights are to be used if an alarm is triggered.

• White Timeout - Use if a white light timer is required. Default is on.

• White Timeout (m) - Set white light timer, for automatic switch off.

• Auto Dimming - When selected the camera will reduce the lamp power, when scenes have too     much light.

• White Exclusion - Option to be used if a white light exclusion area is to be used.

• Power on Park -When the camera powers up, it will go to a preset position.

Low Light Angle - Sets angle when reduced illumination intensity is used.

Low Light Value - Sets illumination intensity

• 20% lamp intensity

• 40% lamp intensity

• 60% lamp intensity

• 80% lamp intensity

• 100% lamp intensity