360 Vision Technology

“Predator HD”

fully functional ruggidised PTZ camera

Predator version detailed within this document;

30x High Definition 1080P Predator camera version

The fully functional rugged PTZ camera shall be a Predator High Definition PTZ camera rated to IP68 with a 30x optical zoom plus 12x digital zoom colour/mono camera.

30x High Definition 720P Night Vision Predator camera version

The fully functional rugged PTZ camera shall be a Predator High Definition PTZ camera rated to IP68 with a 30x optical zoom plus 12x digital zoom colour/mono camera.

20x High Definition 1080P Night Vision ULTRA Predator camera version

The fully functional rugged PTZ camera shall be a Predator High Definition PTZ camera rated to IP68 with a 20x optical zoom plus 12x digital zoom colour/mono camera.

The physical attributes of the unit shall meet the following design and performance

specifications:

- The unit shall be environmentally rated to IP68

- The unit shall operate from +60 degrees to -40 degree centigrade temperature variations      (-50° with optional heater), up to hurricane force wind conditions (above190mph winds)

- The unit shall be machined from 6mm thick aluminum

- The unit shall be classed as vandal resistant

The unit shall have a weather resistant coating in a choice of colors:

- The unit shall be black (RAL 9011)

- The unit shall be white (RAL 9003)

- The unit shall be cool grey (RAL 7035)

- The unit shall be mid grey (RAL 7001)

The unit shall be available in any colour dependant on customer requirement.

Note: Speak to our sales team for clarification and notification of any additional charge

The unit shall have an option of a TEN Year manufacturer’s paint warranty;

Note: Speak to our sales team for clarification of available colour’s and notification of Terms and Conditions and additional charge

- The unit shall have the ability to be mounted upright or inverted

- The unit shall be no more that 7.5Kg in weight

- The unit shall comply with ROHS, CE standards

The unit shall have dimensions of

- 247(W) (Pan Sweep 399)mm

- 410.5(H) (Tilt Sweep 248)mm

- The unit shall feature an optically perfect, flat viewing window

- All weatherproof seals within the unit shall last for the 3yr warranty period of the unit without need for maintenance or replacement

- The unit shall be supplied with attached 4 inch PCD base as standard with pre-drilled hole suitable for a safety chain

- The unit shall be capable of full operation at a composite cable lengths of up to 40 meters maximum between PTZ camera head and the power supply

- The unit’s pan and tilt system shall have a minimum performance or exceed the following

- design:

- The unit shall feature 3 Phase motors for its pan/tilt drive

- The pan drive speed shall range from 0.1º per second to 120º per second, 360° continuous rotation and absolute positioning

- The tilt drive speed shall range from 0.1º per second to 120º per second and absolute positioning

- The unit shall be capable of 180° Tilt coverage

- The unit shall be capable of 360º continuous rotation pan

- The unit shall utilise optical sensor positioning technology

- The unit shall have absolute pan and tilt position accuracy

Connectivity options of the Predator shall meet the following design and performance

specifications:

- Connections to Predator with 12way Pin Out

The Predator shall have a purpose manufactured composite cable available in the following lengths;

- 3m

- 10m

- 25m

- 40m

The composite cable contains video coax, power and data cables and shall be capable of full operation at a composite cable lengths of up to 40 meters maximum between PTZ camera head and the power supply

The unit shall have one end fitted with a IP68 12-way connector for direct connection to the Predator, the other end will be supplied with all cables stripped and tinned ready to be connected to the two–part connectors located within the Predator power supply, a RJ45 output will be integral to the power supply unit enabling connection via an RJ45 data cable to a IP network.

Connections to Predators with HMA

Predator can be manufactured with a HMA (Hinged Mount Adaptor).

When manufactured with an HMA the ‘Predator Composite Cable’ is not used, the HMA allows the use of conventional CAT5, CAT5e and CAT6 cables .

The Ethernet connection uses standard CAT5, CAT5e or CAT6 cables fitted with RJ45 connectors.

| RJ45 | CAT5, CAT5e, CAT6 Cable | Signal |
| --- | --- | --- |
| Pin 1 | White with Orange Band | Tx+ |
| Pin 2 | Orange | Tx- |
| Pin 3 | White with Green Band | Rx+ |
| Pin 4 | Blue |  |
| Pin 5 | White with Blue Band |  |
| Pin 6 | Green | Rx- |
| Pin 7 | White with Brown Band |  |
| Pin 8 | Brown |  |

Access to the connections are made by tilting the Predator.

A lanyard will be fitted at point of manufacture to the HMA.

24Vac Power connections between Predator PSU and Predator that is fitted with HMA

| Cable Size | Predator (No Lamps) |
| --- | --- |
| 0.75mm | 20m |
| 1.00mm | 25m |
| 1.25mm | 30m |
| 1.50mm | 40m |
| 2.50mm | 65m |
| 3.00mm | 80m |
| 4.00mm | 100m |

The camera module shall meet and/or exceed the following design and performance specifications:

30x HD 1080P camera module

- Shall feature a Sony 1/ 2.8-type Exmor CMOS module

- Shall be a colour/monochrome unit

- Shall switch from day mode (colour) to night mode (monochrome) via  the cameras module 360 vision software, it shall also be user selectable via preset levels, or via the Predators OSD Menus

- Shall have a minimum resolution of 2.38 Megapixels

- Shall have an 30x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.35 lux – colour

- minimum illumination sensitivity of 0.013 lux – monochrome

Codec Features - PAL

- Multi Codec 3 Streams

- Image compression – H.264, MPREG4, MJPEG

- HD 1080p – 1920 x 1080

- HD 720p – 1280 x 720

- Resolution Full – 720 x 576

- Resolution Half – 720 x 288

- Resolution CIF – 352 x 288

- Resolution QCIF – 176 x 144

- Protocols – ONVIF 2.2 (S), TCP/IP, HTTP, RTP,RTSP, 360SDK

- Compression levels – CBR, VBR, Adjustable up to 12Mbps

- Frame rate – up to 25fps

- Security – Encryption AES-128 (ECB Mode) User Name and Password

30x HD 720P Night Vision camera module

- Shall feature a Sony 1/ 3-type Exmor CMOS module

- Shall be a colour/monochrome unit

- Shall switch from day mode (colour) to night mode (monochrome) via  the cameras module 360 vision software, it shall also be user selectable via preset levels, or via the Predators OSD Menus

- Shall have a minimum resolution of 1.37 Megapixels

- Shall have an 30x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.25 lux – colour

- minimum illumination sensitivity of 0.008 lux – monochrome

Codec Features - PAL

- Multi Codec 3 Streams

- Image compression – H.264, MPREG4, MJPEG

- HD 720p – 1280 x 720

- Resolution Full – 720 x 576

- Resolution Half – 720 x 288

- Resolution CIF – 352 x 288

- Resolution QCIF – 176 x 144

- Protocols – ONVIF 2.2 (S), TCP/IP, HTTP, RTP,RTSP, 360SDK

- Compression levels – CBR, VBR, Adjustable up to 12Mbps

- Frame rate – up to 25fps

- Security – Encryption AES-128 (ECB Mode) User Name and Password

20x HD 1080P Night Vision ULTRA camera module

- Shall feature a Sony 1/ 2.8-type Exmor CMOS module

- Shall be a colour/monochrome unit

- Shall switch from day mode (colour) to night mode (monochrome) via  the cameras module 360 vision software, it shall also be user selectable via preset levels, or via the Predators OSD Menus

- Shall have a minimum resolution of 2.14 Megapixels

- Shall have an 20x optical zoom with 12x digital zoom

The camera module shall have

- minimum illumination sensitivity of 0.1 lux – colour

- minimum illumination sensitivity of 0.008 lux – monochrome

Codec Features - PAL

- Multi Codec 3 Streams

- Image compression – H.264, MPREG4, MJPEG

- HD 1080p – 1920 x 1080

- HD 720p – 1280 x 720

- Resolution Full – 720 x 576

- Resolution Half – 720 x 288

- Resolution CIF – 352 x 288

- Resolution QCIF – 176 x 144

- Protocols – ONVIF 2.2 (S), TCP/IP, HTTP, RTP,RTSP, 360SDK

- Compression levels – CBR, VBR, Adjustable up to 12Mbps

- Frame rate – up to 25fps

- Security – Encryption AES-128 (ECB Mode) User Name and Password

The unit shall have the following operational design and performance specifications

- 360 telemetry presets

- Preset titles of 20 characters for each preset

- 360 telemetry presets

- Preset titles of 20 characters for each preset

- 4 programmable tours (maximum of 90 presets per tour)

- 4 learned patrols, each with up to 30 minute duration.

- Up to 24 Privacy Zones

- Digital Zoom - Use this option to enable or disable digital zoom.

- Wide Dynamic - Enable WDR

- Sunshield - Reduces tilt range of the camera for when a sunshield is fitted.

- Auto Flip - Use this option to set how the camera behaves when full tilt down is reached.

- Proportional - Automatically reduces/increases pan/tilt speed depending on zoom ratio.

- Freeze Presets - This option freezes the image, when moving between presets.

- Attack Detect - Camera will re-initialise camera motors so that camera looks back to the original position, if the camera is physically forced/moved. Default is on.

- Upright Mount - Can be used to flip the image, so the camera can be used on a pole or a ceiling.

- Sensor Frame Rate - Can be used to set the camera to suit 25fps or 30fps

- Power on Park - When the camera powers up, it will go to a preset position

- Near Focal Limit - Can be used to set the near focal limit

- Pan/Tilt Speed - Can be used to set the pan/tilt speed.

Home Function

- Preset 1 to 360

- Tours 1 to 4

- Mimics 1 to 4

- None

Engineering Servicing Menu

- Power Reset

- Factory Reset

The unit shall have the following operational Network design and performance specifications

DHCP Enable -

• IP Address - Input a static IP address, if DHCP is not used.

• Netmask - Input the Network Subnet Mask.

• Default Gateway - Input the Network Gateway.

• Primary Name Server - Input the Domain name server, if required.

• MTU Size - Option to change the MTU size, default is 1500 bytes.

RTSP Tab

• Multicast Stream 1 - Set IP address and port for stream 1.

• Multicast Stream 2 - Set IP address and port for stream 2.

Unicast

If lots of receivers (users) want to receive video streams from the Predator-HD, each receiver will have a separate video stream. This would increase the network bandwidth used. E.g. five users would require five video streams that typically will require five times the bandwidth.

Multicast

If the receivers of the video streams are suitable for using ‘multicast’ and the video streams will all have the same format, then it may be possible to use ‘multicast’. In this mode, the CODEC in the Predator-HD chooses a ‘new UDP address’ (Typically an address which is outside the normal range of addresses for the network to which the Predator-HD is connected) and sends a video stream to the ‘new UDP address’. Multiple receivers can then access this single video stream. Consequently the bandwidth that is required by the Predator-HD is typically reduced to the single video stream.

The unit shall have the following electrical design and performance specifications:

Total power consumption of the standard camera shall be no more than 25.2W and no more than 50W when fitted with the heater option

Predator power supply providing +24V ac

An optional slim line power supply, for fitting in restricted spaces, providing +24V AC from the following input voltages 115V AC or 240V AC

Camera unit shall be fitted with control PCB’s including a control processor, voltage regulator, 3 Phase motor drives and connectivity of ancillary items such as heaters, infrared illuminators, washer/wiper and speakers

The unit shall have a range of options and accessories available for specific mounting and operational considerations:

- Suitable accessories shall be available for the unit to be mounted in an upright or inverted position, either on a base, from a ceiling, on a wall or on at the corner of a structure

Option to fit a washer nozzle to the 4” PCD base

- Option to fit a heater contained within the camera head & body

Optional integrated wiper with replaceable wiper blade

- Optional safety chain

Optional 8 input alarm card (incl. washer pump drive function) housed in the PSU

Optional washer pump drive card (no alarm inputs) housed in the PSU

The unit shall have the following support information and equipment shipped with it:

Comprehensive operation manual and installation manual (in disk format)

Comprehensive quick start guide (in disk format)